EXECUTIVE SUMMARY

Comprehensive Bill of Materials — 100 Building Configurations

February 2026  |  Prepared for Bobby Burnett & Josh Meunier

# Project Scope

This deliverable contains 100 individual Bills of Materials covering every standard building configuration offered by 13|7 FrameWorks. The BOMs span 2 construction types (Pole Barn weld-up and I-Beam rigid frame), 10 building sizes (1,200 to 6,000 sq ft), and 5 eave heights (10’, 12’, 14’, 16’, and 20’). All configurations are shell-only packages at 3:12 roof pitch, with material quantities calculated to pass a Texas municipal building inspection. Each BOM carries the company’s target 45% gross margin uniformly across all line items.

The 100 BOMs are organized into 10 Excel workbooks (one per building size), each containing 10 tabs—five Pole Barn heights and five I-Beam heights—plus a Summary tab. Across all files, 12,600 live Excel formulas drive every cost, selling price, and margin calculation, ensuring the spreadsheets update dynamically if any unit cost is changed.

# Key Findings

## Height Is the Dominant Cost Driver

Moving from a 10’ to a 20’ eave height increases material cost by 43–78% depending on building size and type. Smaller buildings see the largest percentage increase because the height-driven components (taller posts, additional girt rows, taller wall panels, portal bracing, fly braces) represent a larger share of total cost. At 14’+ eave height, the BOM expands from 28 to 30 line items with the addition of fly braces and sag rods. At 16’+, it grows to 32 items with portal frame bracing kits and DTI washers for wind resistance.

## Cost Impact: 10’ vs. 20’ Eave Height (Internal Cost)

| Size | Sq Ft | PB 10’ | PB 20’ | PB Δ | IB 10’ | IB 20’ | IB Δ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30'×40' | 1,200 | $10,382 | $17,292 | +67% | $10,192 | $18,097 | +78% |
| 30'×50' | 1,500 | $12,426 | $20,371 | +64% | $12,093 | $21,200 | +75% |
| 40'×40' | 1,600 | $12,739 | $20,477 | +61% | $12,099 | $20,697 | +71% |
| 40'×50' | 2,000 | $15,203 | $24,030 | +58% | $14,285 | $24,099 | +69% |
| 40'×60' | 2,400 | $17,353 | $27,198 | +57% | $16,307 | $27,311 | +67% |
| 50'×50' | 2,500 | $19,113 | $29,052 | +52% | $17,220 | $27,854 | +62% |
| 50'×60' | 3,000 | $21,813 | $32,861 | +51% | $19,642 | $31,511 | +60% |
| 50'×80' | 4,000 | $27,832 | $41,214 | +48% | $24,880 | $39,270 | +58% |
| 60'×60' | 3,600 | $25,788 | $38,004 | +47% | $22,492 | $35,192 | +56% |
| 60'×100' | 6,000 | $40,120 | $57,487 | +43% | $34,512 | $52,511 | +52% |

# Pole Barn vs. I-Beam: The Gap Narrows at Height

At 10’ eave height, Pole Barn material costs run 10.4% higher than I-Beam across all sizes. This gap steadily closes as buildings get taller—at 20’, the premium drops to just 3.4%. The convergence happens because I-Beam’s heavier structural steel (W-shapes vs. 4” tube) becomes proportionally more expensive at taller heights, requiring upgrades from W6×20 to W10×33 columns. Meanwhile, both types share identical panel, purlin, and trim costs which dominate total material spend on taller builds.

## Aggregate Cost Across All 10 Sizes, by Height

| Height | PB Cost | PB Sell | IB Cost | IB Sell | PB Premium |
| --- | --- | --- | --- | --- | --- |
| 10' Eave | $202,769 | $368,675 | $183,722 | $334,044 | 10.4% |
| 12' Eave | $219,338 | $398,800 | $202,775 | $368,684 | 8.2% |
| 14' Eave | $230,451 | $419,002 | $217,062 | $394,660 | 6.2% |
| 16' Eave | $268,235 | $487,698 | $254,071 | $461,951 | 5.6% |
| 20' Eave | $307,986 | $559,974 | $297,742 | $541,350 | 3.4% |

# What Changes at Each Height Tier

10’–12’ (28 items): Standard frame with longer posts, costlier post steel, and one additional wall girt row at 12’. All roof components, purlins, and trim remain unchanged. This is the most cost-effective height range.

14’ (30 items): Adds fly braces (knee braces at every frame for lateral stability) and flange brace sag rod sets (1 per bay). Posts upgrade to heavier wall thickness. Wall panels cost ~40% more than 10’ panels due to added length. Eave strut cost steps up.

16’–20’ (32 items): Adds portal frame bracing kits (critical for wind resistance on tall sidewalls in Texas) and DTI tension-indicating washers on all structural connections. Posts upgrade to 6” tube (Pole Barn) or W10 shapes (I-Beam). Wall girts upgrade from 4” to 6” C-sections. Cross bracing sets increase significantly. At 20’, fly braces double to 4 per frame.

# Recommendation

For customers comparing the two construction methods, the data supports a clear narrative: at standard 10–12’ eave heights, I-Beam offers a 8–10% material cost advantage with superior clear-span capability. At taller heights (16–20’), the cost difference nearly disappears while I-Beam retains its structural superiority for wind and snow loading. Pole Barn remains competitive on faster field assembly time and simpler foundation requirements. For pricing purposes, the height modifier in the estimator app ($0 at 10’, +$1,500 at 12’, +$3,500 at 14’, +$6,000 at 16’, +$12,000 at 20’) aligns directionally with the cost data, though large buildings at 20’ may warrant individual review.

Note: All figures reflect shell-only material packages at 3:12 roof pitch. Costs exclude doors, windows, insulation, concrete, labor, delivery, and accessories. 28–32 line items per BOM depending on height tier. 12,600 formulas across 10 workbooks with 0 calculation errors. Target 45% gross margin applied uniformly.